### Materials and Methods

Image Processing and Preprocessing (Parts 1–3)

Raw Epoxy-based Cyclic Nanobody Immunofluorescence (EpoCyNif) data was acquired as Zeiss .czi files. Initial processing involved illumination correction using BaSiCpy to remove shading artifacts [DOI: 10.1038/ncomms14836]. Subsequently, image tiles underwent stitching and registration using Ashlar [DOI: 10.1093/bioinformatics/btac544], followed by deconvolution [DOI: 10.1364/JOSA.62.000055 & DOI: 10.1086/111605] and Extended Depth of Field (EDF) projection to generate high-quality 2D composite images [DOI: 10.1002/jemt.20092].

To ensure accurate protein quantification, a supervised linear compensation workflow was implemented to correct for signal spillover (channel crosstalk). Correction was restricted to a selected set of donor-target channel combinations where significant spectral overlap was anticipated (Supllemental Table XY). Compensation coefficients were estimated from single-positive regions of interest (ROIs) using a non-negative least squares approach and applied linearly to subtract donor signals from target channels, following the mathematical principles of spectral compensation [DOI: 10.1111/j.1749-6632.1993.tb38775.x]. Autofluorescence (AF) was removed by subtracting non-specific signals derived from dedicated background channels (e.g., AF1/AF2). The contribution of the background signal to each marker channel was estimated using a robust linear regression model (Huber regressor) to minimize the impact of outliers, an approach adapted from the AutoSpill algorithm [DOI: 10.1038/s41467-021-23126-8]. To improve signal perceptibility in heterogeneous tissue areas, images underwent multi-scale adaptive unsharp masking (referred to as ACE in the pipeline). This step enhanced local contrast by amplifying high-frequency components relative to the local mean across multiple spatial scales (radii: 5, 15, 45 px), employing a robust global scaling factor derived from the 98th percentile of pixel deviations[DOI: 10.1109/ICSITECH.2017.8257197].

Segmentation, Feature Extraction, and Quality Control (Parts 4–5)

Segmentation and Feature Extraction (Parts 4–5) Nuclei and cellular boundaries were segmented using the deep-learning-based tool InstanSeg with the fluorescence_nuclei_and_cells model. To enhance segmentation accuracy, a channel-invariant architecture (ChannelNet) was employed, allowing the integration of both nuclear (DAPI) and structural surface markers (e.g., membrane proteins) to robustly define cell boundaries in multiplexed images [DOI: 10.1101/2024.09.04.611150]. Single-cell intensity features were extracted for phenotyping. While mean intensity provides a robust measure for cytoplasmic markers, it can underestimate the abundance of membrane-bound proteins due to signal dilution by non-fluorescent cytoplasmic pixels within the segmentation mask. To address this, we adopted a marker-specific quantification strategy [DOI: 10.1101/2022.06.08.495346]: the 95th percentile (p95) of pixel intensity was calculated for membrane-associated markers (Supplemental Table XY) to robustly capture peak boundary signals, whereas mean intensity was utilized for intracellular, extracellular and nuclear markers. Following feature extraction, the CyLinter pipeline was employed for interactive quality control [10.1038/s41592-024-02328-0]. Subsequent to this quality control step, marker intensity distributions were evaluated to establish specific gating cutoffs for each channel. Based on these validated cutoffs, a combinatorial "subset definition" matrix was applied to systematically assign a specific phenotype to each cell, facilitating precise cell type alignment across the dataset.

In parallel, histological compartments (intestinal crypts) were segmented to enable spatial stratification. For this, we utilized the Micro-SAM framework, a napari-based integration of the Segment Anything Model (SAM) optimized for microscopy applications [DOI: 10.1038/s41592-024-02580-4]. The model (ViT-B backbone) was applied to the Laminin channel to detect the basement membrane enclosing the crypts. To process whole-slide images efficiently, an adaptive tiling strategy with overlap (tile size: 512–1024 px) was implemented. The resulting instance masks were subjected to rigorous morphological filtering: putative crypts were selected based on geometric criteria (minimum area >8,000 pixels; eccentricity <0.9) to exclude artifacts and non-crypt structures, followed by morphological closing and hole-filling operations to ensure mask integrity.

Spatial Analysis and Compartmentalization (Part 6)

Spatial analysis was performed to contextualize cellular phenotypes within the tissue architecture. To ensure morphometric consistency and avoid stereological bias introduced by variable sectioning angles (the "corpuscle problem"), the analysis of intestinal crypts was strictly restricted to orthogonal cross-sections [DOI: 10.1590/S0100-69912012000200010]. Elongated or tangential crypt profiles (longitudinal sections) were identified and excluded based on geometric criteria (eccentricity > 0.9) using the Micro-SAM pipeline. This rigorous selection ensured that cellular frequencies were quantified within geometrically comparable anatomical units. For cellular mapping, single-cell phenotypes were systematically defined based on the quality-controlled feature data exported from the CyLinter pipeline. The spatial coordinates (X,Y centroids) of each phenotyped cell (script 5, cylinter) were then computationally registered with the segmentation masks of the validated crypts. Cells were assigned to histological compartments based on this spatial mapping: cells located within the segmented crypt masks or within a 31-pixel boundary buffer (~10 µm) were classified as "Crypt-associated" (encompassing intraepithelial lymphocytes and peripheral epithelial cells). The Lamina Propria (LP) compartment was defined by strict exclusion: it comprised the total tissue area minus the area of valid crypts, minus the area of excluded longitudinal crypt sections, and minus any regions identified as artifacts or non-mucosal layers. Cell frequencies were quantified either as absolute density (cells/mm²) or normalized to the epithelial cell count (e.g., per 100 epithelial cells).

Statistical Analysis (Part 7)

Statistical analysis was performed to quantify differences in immunological metrics between patient groups. First, the normality of data distributions was assessed using D’Agostino’s K2 test. As all metrics significantly deviated from a normal distribution, non-parametric statistical tests were employed. For comparisons between two independent groups (e.g., Healthy vs. Lynch), the Mann-Whitney U test was applied. For multi-group comparisons (e.g., Healthy vs. MLH1-deficient vs. MSH2-deficient), the Kruskal-Wallis H test was used, followed by Dunn’s post-hoc test with Benjamini-Hochberg correction for multiple testing where appropriate.

To rigorously address the hierarchical structure of the data (multiple crypts nested within individual patients), we applied a summary statistic approach to prevent pseudoreplication [DOI: 10.1186/1471-2202-11-5]. Instead of treating individual crypts as independent observations (which would artificially inflate statistical power), crypt-level data were aggregated to patient-level means prior to hypothesis testing. This method effectively integrates the intra-patient variability (heterogeneity between crypts) into a robust representative value for each biological replicate, ensuring that statistical significance reflects true biological differences between patient groups rather than sampling variance. All statistical tests were two-sided, and a p-value < 0.05 was considered statistically significant.

Data Availability

All computational scripts and Jupyter notebooks used for image processing, segmentation, and statistical analysis are available in the GitHub repository at [Insert GitHub Link].

| Table XY Summary of Computational Modules and Algorithms |  |  |
| --- | --- | --- |
| Pipeline Step | Method / Module | Reference / DOI |
| Illumination Correction | BaSiCpy | 10.1038/ncomms14836 |
| Stitching & Registration | Ashlar | 10.1093/bioinformatics/btac544 |
| Deconvolution | Richardson-Lucy Algorithm | 10.1364/JOSA.62.000055 & 10.1086/111605 |
| Z-Projection | Extended Depth of Field (EDF) | 10.1002/jemt.20092 |
| Spillover Correction | Supervised Linear Compensation (Non-negative Least Squares) | 10.1111/j.1749-6632.1993.tb38775.x |
| Autofluorescence Removal | Robust Linear Regression (Huber) (adapted from AutoSpill) | 10.1038/s41467-021-23126-8 |
| Contrast Enhancement | Adaptive Unsharp Masking (ACE) | 10.1109/ICSITECH.2017.8257197 |
| Cell Segmentation | InstanSeg (ChannelNet architecture) | 10.1101/2024.09.04.611150 |
| Feature Extraction | Marker-specific Quantification (p95 vs. Mean) | 10.1101/2022.06.08.495346 |
| Quality Control | CyLinter | 10.1038/s41592-024-02328-0 |
| Tissue Segmentation | Micro-SAM (Segment Anything Model) | 10.1038/s41592-024-02580-4 |
| Statistical Aggregation | Summary Statistic Approach (Patient-level means) | 10.1186/1471-2202-11-5 |